ODESILANI SMS PRIKAZY Z TERMINALU
Pripojim se terminalem pres tcp/ip
uzivatelske jmeno Admin, heslo 2n

AT!G=A6        Start low-level controlling for SMS (can run only on one port)


Odeslani SMS
AT^SM=0,18,001100098137491318F60000FF05F0B79C2E03,B4         = odeslani SMS bez dorucenky

vyznam: 0 = pujde pres g00
18 = delka PDU - pocita se po dvojicich (po bytech), prvni 00 se nepocita. To znamena, ze spocitam pocet vsech znaku v pdu, vydelim 2 a odectu 1
samotny pdu - 0011 = bez dorucenky, 0031 =  s dorucenkou
B4 = kontrolni soucet. Sectu hexadecimalne Byty (dvojice 00+11+00+09+... a z vysledku vezmu posledni 2 cislice = sectu bez prenosu)


vycteni SMS
AT^SS=0,3
AT^SS=0,2
AT^SS=0,1    Prepinam jednotlivy uloziste a kontroluju, jestli tam neni SMS

Priklad:
AT^SS=0,3 - kontroluju jednu z pameti
*smssel: 0,1 = je tam 1 SMS

Kontrola novych SMS
AT^SX=0        =request to list all SMS messages and status confirmations saved on SIM card.


Priklad:
AT^SX=0    
*smsinc: 0,1,0,0            *smserr (busy,list) or *smsinc (ix=1..255) for each saved SMS or status SMS messages, end of list or empty SIM card - *smsinc (ix=0).
*smsinc: 0,0,0,255


Vycteni SMS
AT^SR=0,1

Priklad:
AT^SR=0,2
*smspdu: 0,2,0,23,0791246030500200040C912470933481610000011052220194400461F45B0D,97
PDU si prevedu v http://rednaxela.net/pdu.php


Smazani SMS
AT^SD=0,1



vysvetleni jednotlivejch Bytu PDU
> 0001010B915121551532F40010104190991D9EA341EDF27C1E3E9743<Ctrl-Z>

Here is what the individual octets represent:

| Size | Value | Description |
| --- | --- | --- |
| 1 octet | 00 | We don’t supply a SMSC number. |
| 1 octet | 01 | PDU type and options. This is a plain SUBMIT-PDU. |
| 1 octet | 01 | Our message reference. |
| 1 octet | 0B | Size of destination telephone number (in digits) |
| 1 octet | 91 | International numbering plan. |
| 6 octets | 5121551532f4 | This represents the destination and it translates to 1 512 555 1234 |
| 1 octet | 00 | Protocol identifier. |
| 1 octet | 10 | Data Coding Scheme.This is the value that cause the message to be a flash message as opposed to a standard SMS. |
| 1 octet | 10 | User Data Length or payload size (in characters). |
| 16 septets | 4190991D9EA341EDF27C 1E3E9743 | The payload, also known as User Data. In this case it is just GSM encoded text. This particular string represents “A flash message!”” |

A more elaborate explanation of the GSM encoding and other fields is giving in a previous post in this series.

Vynatek z manualu SG

| SMS Control | SMS control can be activated for only one session at a time |
| --- | --- |
| AT!G=# | A6 – activate control via used session 55 – deactivate control via used session 99 – deactivate control on all sessions |
| AT^MS=ch | Module status ch=0 – 31 |
| *module: ch,sim,sts3,sts2,phs2,type | Response to AT^MS: ch – number of wireless engine sim – number of currently used SIM: 0-none, 1-8-holder 1-8 sts3 – status of layer 3 sts2 – status of layer 2 phs2 – substatus of layer 2 type – type of wireless engine |
| AT^MI=ch | Module info ch = 0 -31 |
| *minfo: ch,sim,netid,”oper”,new,cnt | Response to AT^MI netid – network ID of active wireless network oper – name of active wireless network new – number of received and saved SMS on SIM card cnt – total number of sent SMS messages according to statistics |
| *smsinc: ch,ix,sts,mem | SMS message has been received and saved ix – index number of saved SMS (0-250) sts – status of SMS (from +CMGL: ix.sts.len) mem – type of used storage (1,2,3) |
| *smsrep: ch,ix,sts,mem | SMS status confirmation has been received sts – reserved (0) |
| *smsout: ch[/id],ref,req | SMS message was sent and not saved to SIM card Ref – reference number of SMS (0-255) – to be used in status confirmation messages ID – third party identification number of SMS |
| *smspdu: ch,ix,sts,len,pdu,csum | Content of SMS message len – length of SMS message (number of bytes in PDU) PDU – content of message in PDU format csum – checksum of all PDU bytes (2 hexa digits) calculated without carry |
| *smsdel: ch,ix | SMS message deletion confirmation |
| *smserr: ch[/id],ix,req,err,cms | Fault command response, or SMS not sent req – required wireless engine or GSM group err – error code (6-busy,40-write,41-read,42-delete,43-list) cms – error cause from wireless engine |
| *smssel: ch,cnt | SMS storage change confirmation |
| AT^SX=ch | SMS listing - request for listing of all SMS messages and status confirmations saved on SIM card. Possible answers:: *smserr (busy,list) or *smsinc (ix=1..255) for each saved SMS or status SMS , end of list or empty SIM card - *smsinc (ix=0). |
| AT^SR=ch.ix | SMS read - request for reading of an SMS message or SMS status saved on SIM card. Possible answers: *smserr (busy,read) or *smspdu |
| AT^SD=ch,ix | SMS delete - request for deletion of an SMS message (or SMS status message). Possible answers: *smserr (busy,delete) or *smsdel |
| AT^SM=ch,len,pdu,chsum | SMS to module - request for sending a message via GSM module 0..31 or via any GSM module (ch=32). Possible answers: *smserr (busy,write) or *smsout |
| AT^SG=grp[/id],len,pdu,csum | SMS to group - request for sending an SMS message via GSM group 1..8. Possible answers: *smserr (busy,write) or *smsout |
| AT^SS=ch,mem | Command for changing SMS storage. Mem (1;2;3) = type of SMS storage Siemens: 1=SM, 2=ME, 3=ME Ericsson: 1=SM, 2=ME, 3=ME Wavecom: 1=SM, 2=ME, 3=SR |